# Initial Data

## Geometry Data

| Height of structure | h | 12 | m |
| --- | --- | --- | --- |
| Depth of underground facing panel | ht | 1,75 | m |
| Horizontal distance between reinforcing elements | t | 0,75 | m |
| Verical distance between reinforcing elements | f | 0,75 | m |

## Soil properties

| Unit weigt of backfill | ! | 20 | kN/m3 |
| --- | --- | --- | --- |
| Internal friction angle of coarse backfill | "k | 29 | ° |
| Cohesion of coase backfill | c’k | 0 | kPa |
| Friction angle between coarse backfill and steel reinforcing strips | µ | 0,37 |  |
| cohesion of subsoil | c’k,A | 80 | kPa |

## Loads

| Characteristic value of fill weight | gk | 30 | kN/m2 |
| --- | --- | --- | --- |
| Characteristic value of vehicle load | qk | 24 | kN/m2 |

## Other data

| Steel quality |  | S275 | kN/m2 |
| --- | --- | --- | --- |
| Corrosion coefficient of steel reinforcing strips | K1 | 0,5 |  |
|  | K2 | 30 |  |
|  | N | 0.8 |  |
| Design lifetime | # | 50 | years |

# Critical failure surface parameters

With the given data of the internal friction (!k) of 38°, a linal interpolation is done between !k= 35° and !k=40° to get the value of the critical parameter value.

| "k | " | D/h | Kc | Kq |
| --- | --- | --- | --- | --- |
| 25° | 38° | 0.43 | 0,26 | 0,70 |
| 30° | 43.5° | 0,37 | 0,21 | 0,60 |
| 29° | 39.1° | 0,382 | 0,22 | 0,62 |

## Data for Critical failure surface

| Earth pressure coefficient from max. normal force | Kc | 0.22 |  |
| --- | --- | --- | --- |
| Earth pressure coefficient from surface | Kq | 0.62 |  |
|  | D = D/h*h | 4.584 | m |
|  | ε = 90°- (ω − φ) | 21.9 | ° |
| Length of reinforcing strips | l = 0,8*h | 9.6 | m |

Design method:

- Measure distance D from the top point of the structure. From the endpoint of this distance measure angle φ .

- Measure angle $ from the base point of the structure.

- The intersection of the two lines is point O.

- Distance Ra and Rb can be measured from the drawing.

- Points of the logarithmic spiral failure surface can be calculated by using the

formular: R! 	e#!∗%&'

- The " angle needs to be in radians while the φ must be in [°].

- The logarithmic spiral failure surface can be constructed by determining Ri values every 5° from 0° to ".

The results from this process are as follows:

| ω | Ri |
| --- | --- |
| 0 | 1384.5 cm |
| 5 | 1452.5933 cm |
| 10 | 1524.5862 cm |
| 15 | 1600.1472 cm |
| 20 | 1679.4531 cm |
| 25 | 1762.6896 cm |
| 30 | 1850.0514 cm |
| 35 | 1941.7429 cm |
| 39.1 | 2016.5 cm |

# Determination of earth pressures

In case of unloaded surface horizontal earth pressure (*h, B) increases linearly until depth H0, then it remains constant until the base level. In case of loaded surface horizontal earth pressure (*h,A) decreases linearly until H0 depth, then it remains constant until base level.

*h,A and *h, B stress need to be calculated at the level of every reinforcing element, then the covering curve (*h) need to be constructed.

| Calculation oft he Kcs coefficient of earth pressure | Kcs | 0.6194 |  |
| --- | --- | --- | --- |
| Depth associated with the maximum reinforcement force (limit depth) | H0 | 4.2623 | m |
| Value of lateral earth pressure at the surface | σh,A,F | 36.456 | kN/m2 |
|  | σh,B,F | 22.32 | kN/m2 |
| Value of lateral earth pressure at the limit depth | σh,A,H0 | 18.7542 | kN/m2 |
|  | σh,A,H0 | 22.5051 | kN/m2 |

The horizontal earth pressure at each reinforcement level were measured from the earth pressure diagrams drawn in AutoCAD.

# Analysis of sliding

The reinforced retaining wall should be considered as a single gravity wall.

| Coefficient of active earth pressure | Ka | 0.2692 |  |
| --- | --- | --- | --- |
|  | β | 0 | ° |
|  | δ = φ | 29 | ° |
|  | - | 0 | ° |
| The characteristic value of earth pressures due to self weight | Pa,g,h,k | 387.6854 | kN/m |
| The characteristic value of earth pressures due to uniformly distributed surface load: | Pa,q,h,k | 77.5371 | kN/m |
| The design value of sliding force Ed | Ed | 628.0503 | kN/m |

## The design value of resistance

| If sliding on cohesive soil surface | Rd | 698.1818 | kN/m |
| --- | --- | --- | --- |
| If sliding on granular soil surface | Rd | 120.9402 | kN/m |
|  | Gk | 240 | kN/m |

The smallest resistance is taken: Rd = Rd =  120.9402 kN/m  < Ed = 628.0503 kN/m

The retaining wall is not sufficiently secured against sliding.

# Analysis of Base Failure

Analysis of bearing capacity

| The characteristic value of resistance | Rk | 411.3274 | kN/m |
| --- | --- | --- | --- |
| The design value of resistance | Rd | 293.8053 | kN/m |
| The design value of the load on the base of the retaining wall (effect of action) | Vd | 396.9 | kN/m |

Rd = 293.8053	kN/m 	 > V d  = 396.9 kN/m

The bearing capacity of the structure is not satisfactory.

# The maximum height of the structure

| The maximum height of the structure | hmax | 8.1817 | m |
| --- | --- | --- | --- |

Earthworks – Projekt 1

# Horizontal earth pressure

| level |  | level of reinforcing strips | Horizontal earth pressure | vertical stress | safty factor against pull out |  | sufficiency |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | oh | ovi | k1 |  |  |
|  |  | [m] | [kN/m2] | [kN/m2] |  |  |  |
|  | 1 | 0 | 36.456 | 30 |  | 0 | sufficient |
|  | 2 | 0,375 | 34.8708 | 37.5 |  | 1.42 | sufficient |
|  | 3 | 1,125 | 31.7004 | 52.5 |  | 1.64 | sufficient |
|  | 4 | 1,875 | 28.53 | 67.5 |  | 1.57 | sufficient |
|  | 5 | 2,625 | 25.3596 | 82.5 |  | 1.76 | sufficient |
|  | 6 | 3,375 | 22.499 | 97.5 |  | 1.81 | sufficient |
|  | 7 | 4,125 | 22.5051 | 112.5 |  | 2.15 | sufficient |
|  | 8 | 4,875 | 22.5051 | 127.5 |  | 1.69 | sufficient |
|  | 9 | 5,625 | 22.5051 | 142.5 |  | 1.97 | sufficient |
|  | 10 | 6,375 | 22.5051 | 157.5 |  | 2.28 | sufficient |
|  | 11 | 7,125 | 22.5051 | 172.5 |  | 2.63 | sufficient |
|  | 12 | 7,875 | 22.5051 | 187.5 |  | 3.01 | sufficient |
|  | 13 | 8,625 | 22.5051 | 202.5 |  | 3.44 | sufficient |
|  | 14 | 9,375 | 22.5051 | 217.5 |  | 3.92 | sufficient |
|  | 15 | 10,125 | 22.5051 | 232.5 |  | 4.45 | sufficient |
|  | 16 | 10.875 | 22.5051 | 247.5 |  | 5.04 | sufficient |
| 17 |  | 11.625 | 22.5051 | 262.5 |  | 5.7 | sufficient |
| 18 |  | 12 | 22.5051 | 277.5 |  | 6.06 | Sufficient |

Earthworks – Projekt 1

# Thickness of reinforcing strip

| Level |  | Level of reinforcing strips | Thickness of reinforcimg strip | Increased width from corrosion | Calculated thickness of reinforcing strip | applied thickness of reinforcing strip |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | vi | vk | vcpl,i | vapl,i |
|  |  | [m] | [mm] | [mm] | [mm] | [mm] |
|  | 1 | 0 | 0.5 | 5.4928 | 5.99 | 6 |
|  | 2 | 0,375 | 0.48 | 5.4928 | 5.97 | 6 |
|  | 3 | 1,125 | 0.58 | 5.4928 | 6.08 | 7 |
|  | 4 | 1,875 | 0.79 | 5.4928 | 6.28 | 7 |
|  | 5 | 2,625 | 0.88 | 5.4928 | 6.37 | 7 |
|  | 6 | 3,375 | 1.03 | 5.4928 | 6.53 | 7 |
|  | 7 | 4,125 | 1.04 | 5.4928 | 6.53 | 7 |
|  | 8 | 4,875 | 1.55 | 5.4928 | 7.05 | 8 |
|  | 9 | 5,625 | 1.55 | 5.4928 | 7.05 | 8 |
|  | 10 | 6,375 | 1.55 | 5.4928 | 7.05 | 8 |
|  | 11 | 7,125 | 1.55 | 5.4928 | 7.05 | 8 |
|  | 12 | 7,875 | 1.55 | 5.4928 | 7.05 | 8 |
|  | 13 | 8,625 | 1.55 | 5.4928 | 7.05 | 8 |
|  | 14 | 9,375 | 1.55 | 5.4928 | 7.05 | 8 |
|  | 15 | 10,125 | 1.55 | 5.4928 | 7.05 | 8 |
|  | 16 | 10,875 | 1.55 | 5.4928 | 7.05 | 8 |
|  | 17 | 11.625 | 1.55 | 5.4928 | 7.05 | 8 |
| 18 |  | 12 | 1.55 | 5.4928 | 7.05 | 8 |

| Level | Level of reinforcing strip | Normal force(Charactaristic value) | Normal force design value | Distance | Cohesion length | Width of steel reinforcing strip | Applied width of steel reinforcing strip |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Nik | Ni,d | Di | D-Di | Si | Sapl |
|  | (m) | (kN) | (kN) | (m) | (m) | (cm) | (cm) |
| T1 | 0 | 20.5065 | 27.6838 | 4.59 | 5.01 | 20 | 20 |
| T2 | 0.375 | 19.6148 | 26.48 | 4.59 | 5.01 | 20 | 20 |
| T3 | 1.125 | 17.8315 | 24.0725 | 4.59 | 5.01 | 13.6 | 15 |
| T4 | 1.875 | 16.0481 | 21.665 | 4.54 | 5.06 | 9.4 | 10 |
| T5 | 2.625 | 14.2648 | 19.2575 | 4.45 | 5.15 | 6.7 | 8 |
| T6 | 3.375 | 12.6411 | 17.0654 | 4.32 | 5.28 | 4.9 | 6 |
| T7 | 4.125 | 12.6557 | 17.0852 | 4.15 | 5.45 | 4.1 | 6 |
| T8 | 4.875 | 12.6591 | 17.0898 | 3.94 | 5.66 | 3.5 | 4 |
| T9 | 5.625 | 12.6591 | 17.0898 | 3.695 | 5.905 | 3 | 4 |
| T10 | 6.375 | 12.6591 | 17.0898 | 3.41 | 6.19 | 2.6 | 4 |
| T11 | 7.125 | 12.6591 | 17.0898 | 3.085 | 6.515 | 2.3 | 4 |
| T12 | 7.875 | 12.6591 | 17.0898 | 2.725 | 6.875 | 2 | 4 |
| T13 | 8.625 | 12.6591 | 17.0898 | 2.33 | 7.27 | 1.7 | 4 |
| T14 | 9.375 | 12.6591 | 17.0898 | 1.89 | 7.71 | 1.5 | 4 |
| T15 | 10.125 | 12.6591 | 17.0898 | 1.415 | 8.185 | 1.3 | 4 |
| T16 | 10.875 | 12.6591 | 17.0898 | 0.89 | 8.71 | 1.2 | 4 |
| T17 | 11.625 | 12.6591 | 17.0898 | 0.315 | 9.285 | 1 | 4 |
| T18 | 12 | 12.6591 | 17.0898 | 0 | 9.6 | 0.98 | 4 |